ME326 - INTERMEDIATE FLUID MECHANICS

EXAM 1

NAME: __________________________________

STUDENT NUMBER: _____________________

SOLVE BOTH PROBLEMS

| PROBLEM | MARK |
| --- | --- |
| 1 |  |
| 2 |  |
| TOTAL |  |

Problem 1 (50 points)

uUo=53yδ−23(yδ)5/20<y<δ1y > δ  For a boundary layer over a flat plate assume that

Use the von Karman momentum integral to determine

a)  Boundary layer thickness.

b)  Momentum thickness.

c)  Local skin friction coefficient Cf.

d)  Drag coefficient CD.

Problem 2 (50 points)

A viscous fluid is between two concentric				 2

cylinders of radii a and b, which are rotating

respectively, at constant angular velocities

and ω2 in the counterclockwise direction as

shown.  Assume an axisymmetric, steady flow 						a

for which there is no axial velocity.

a) Write down the differential equations of						  b

motion in its simplest form.

b) Specify the boundary conditions.

c) When b = 2a and ω1 = ω2 = ω, obtain the

velocity profile as a function of r, ω and a.

d) Given that pressure at r = a is Po, determine

the pressure as a function of r, ω and a.

Navier-Stokes Equation

Cartesian	ρ(u∂u∂x+v∂u∂y)=−∂p∂x+μ(∂2u∂x2+∂2u∂y2)+ρgx

ρ(u∂v∂x+v∂v∂y)=−∂p∂y+μ(∂2v∂x2+∂2v∂y2)+ρgy

∂u∂x+∂v∂y=0

Cylindrical

Continuity	1r∂(rvr)∂r+1r∂vθ∂θ+∂vz∂z=0

Conective Time Derivative	V⋅∇=vr∂∂r+1rvθ∂∂θ+vz∂∂z

Laplacian operator		∇2=1r∂∂r(r∂∂r)+1r2∂2∂θ2+∂2∂z2

The r-momentum Equation

ρ(∂vr∂t+V⋅∇vr−1rvθ2)=−∂p∂r+μ(∇2vr−vrr2−2r2∂vθ∂θ)+ρgr

The -momentum Equation

ρ(∂vθ∂t+V⋅∇vθ+1rvrvθ)=−1r∂p∂θ+μ(∇2vθ−vθr2+2r2∂vr∂θ)+ρgθ

The -momentum Equation

ρ(∂vz∂t+V⋅∇vz)=−∂p∂z+μ∇2vz+ρgz

Boundary Layer

τw=ρU2dθdx,		D(x)=ρU2θ,	   θ=0δuU(1−uU)dy, 	CF=τw12ρU2=2dθdx

δ=0δ(1−uU)dy,	CD=D12ρU2L=2θ(L)L