ME326 - INTERMEDIATE FLUID MECHANICS

EXAM 3

NAME: __________________________________

STUDENT NUMBER: _____________________

SOLVE BOTH PROBLEMS

| PROBLEM | MARK |
| --- | --- |
| 1 |  |
| 2 |  |
| TOTAL |  |

Problem 1 (50 points)

Consider a converging-diverging nozzle with a throat area of 0.0015 m2 and exit area of 0.002 m2.   The stagnation pressure is 600 kPa and stagnation temperature is 10C.

a)  When there is shock at section S having an area of 0.0018m2, find the pressure and Mach number after the shock and at the exit.

b) When there is no shock and the exit pressure is 580 kPa, find the velocity and Mach number at the throat. 							     S

Po=600 kPa

To=10 C					       Pe

Shock

Problem 2 (50 points)

Air enters a constant area duct at a velocity of 520.78 m/s with a pressure of 50 kPa and a temperature of 300 K.  The diameter of the duct is D = 0.2 m and the friction coefficient is f = 0.02.  A Shock is present at a distance of 0.8 m from the inlet.  The exit pressure is atmospheric with P4=101.3kPa. Determine,

a)  M1, M2, M3, P2and P3.			     0.8 m			  L34

b) M4and the L34.

1				2     3		        4