# ME 639				Midterm Exam

- (20 Points) Consider the turbulent flow of an incompressible fluid.  Estimate the order of magnitude of the following quantities in terms of u, λ, and Λ:

a)  ωi'ωj'Dij b) ∂Ui∂xk∂uj'∂xk c) ∂2ωj'∂xj∂xk∂2ωi'∂xi∂xk

d) ωk'∂ωk' ∂xi e)  ∂2ui'∂xj∂xk∂2ui'∂xj∂xk f) uj'ωi'uj'ωi'

- (30 Points) The equations for hear and mass transfer are given as

∂T∂t+uj∂T∂xj=α∂2T∂xj∂xj,       ∂C∂t+uj∂C∂xj=α∂2C∂xj∂xj

Here α is the diffusivity which is assumed to the same for heat and mass.  Derive the transport equations for the fluctuation temperature, T’, and fluctuation concentration C’.   Evaluate temperature-concentration correlation,T'C'.   Assume the fluid is incompressible.  Identify the terms corresponding to production, dissipation, diffusion and convection.  Find the order of magnitude of different terms.

- (25 Points) For a laminar boundary layer flow over a flat plate, obtain the variations of Us and  with x.  The governing equation are

U∂U∂x+V∂U∂y=ν∂2U∂y2

∂U∂x+∂V∂y=0

4.	(25 Points) For stirring a cup of coffee with a 1 cm spoon requires a power of 0.0005 Watts.  Given that the mass of the coffee is 0.05 kg and the kinematic viscosity of water (coffee) is ν=10−6m2s, estimate the Kolmogorov length and time scales, the turbulence fluctuation energy and time scale as well as the Taylor scale in the cup.