# ME 639					HW Set 4

- For a pipe of radius ro show that

−u'v'+νdUdy=u21−yro

y	        ro

- Show that the maximum dimensionless production in a turbulent near wall flow is 0.25.  That is

Max−u'v'u2dU+dy+=0.25

- For Rannie's eddy viscosity model, νTν=sinh2k1y+.  Show that the corresponding mean velocity field is given by  U+=1k1tanhk1y+.

- Show that near a solid wall, as y→ 0, −u'v'≈ Ay3+By4+...., where  A=14(∂u'∂y)2+12∂u'∂y∂2w'∂y∂z.

- Show that the mixing length model follows from the assumption of local equilibrium (production = dissipation).   (Hint: νT=k12ℓ,−u'v'≈ CD12k).

- Develop a two-equation type model for the mean and mean-square temperature fluctuation in a boundary layer flow.  (Hint: follow the steps similar to those for k−ε model.)

- For compatibility of k and z equations for decay of turbulence behind a grid show that C2=CD(m−n2).  (Hint: z=kmℓn, νT=k12ℓ)

- For the modified Burger's equation given as ∂u∂t+u∂u∂x+βνu=0, find the transport equation for the evolution of the joint pdf of u and x.  Show that no closure assumption is needed.  (Hint: Restate the equation in Lagrangian form, dudt=−βνu, dxdt=u.)